Design Verification Test (DVT) Plan

ESP32-S3-WROOM-1-N8 Development Board — Rev A

| Field | Value |
| --- | --- |
| Board | ESP32-S3 dev board, 76.2 × 25.4 mm, 4-layer |
| Fab data | Gerber / BOM / Pick-and-place / flying-probe exports dated 2026-06-28 |
| Assembler | JLCPCB (LCSC parts) |
| Doc version | 1.2 — 2026-07-21. Scope reconciled with the executed DVT tracker (first-article boards); RainMaker BLE provisioning and Arduino OTA descoped for Rev A. Supersedes v1.1 (2026-07-03). |
| Board S/N under test | ____________ (use ESP32 MAC — see MCU-03) |
| Tester / Date | ____________ / ____________ |
| Test firmware version | ____________ |
| Companion results file | ESP32_RevA_DVT_Tracker.xlsx — per-measurement results, pass/fail rollup, and issue log for the boards actually built. |

# Table of Contents

# 0.  How to Use This Document

Run sections in order. Sections VIS and PRE are unpowered and are gates: any failure there means do not apply power until it is resolved. First power-up (PWR-01) must use a current-limited bench supply, not a USB port on your computer.

Each test has an ID, procedure, expected result, and blank Measured / P/F fields. Record actual numbers, not just pass/fail — measured values are what make the first article comparable to unit #50 later. Detailed executed results for the first-article boards live in the companion tracker; §20 carries the rollup.

Stop-on-fail rules:

- Any rail-to-GND short in PRE → stop.

- Current limit trips at first power-up → stop and thermal-inspect.

- Battery tests: never connect a Li-Po until the unpowered battery-pin no-short check (PRE-11) and the reverse-polarity test (BAT-01, done with a bench supply) have passed.

## Rev A scope reconciliation

This revision documents the test campaign that was actually executed on the first-article boards. The following planned items were descoped for Rev A and removed from the tables below; their IDs are intentionally left as gaps so existing references stay stable:

- RF-B-02 / RF-B-03 / RF-B-04 (RainMaker BLE provisioning, cloud reporting, WiFi+BLE coexistence) — RainMaker was dropped from the product. The shipping demo provisions over a WiFi captive portal and has no cloud dependency, so coexistence is moot. RF-B-01 (BLE radio RSSI) is retained for board QA.

- SYS-03 (Arduino OTA update) — descoped for Rev A.

- PRE-04 (BATT+→GND resistance), BAT-03 / BAT-04 (over-voltage inhibit, overcurrent trip — optional first-article stress tests), BAT-08 (runtime baseline — superseded by the PPK2 power characterization).

- OLED-06 / OLED-07 / OLED-08 (charge-pump VCC / VCOMH / current measurements), RGB-03 (full-white current), ADC-03 (battery % vs DMM), RF-W-05 / RF-W-06 (throughput, range walk) — display/RGB/ADC verified functionally; RF verified by scan, RSSI, association, and ping.

A few expected-value limits were also tightened to the values recorded during execution (noted inline). Deviations found during DVT are listed in §20 and in the tracker issue log.

# 1.  References and Revision Notes

Source data used to build this plan: BOM_Board1_PCB1_20260628.xlsx, PickAndPlace_PCB1_2026_06_28.xlsx, FlyingProbeTesting.json (net-to-pad map), the Gerber set (board outline 76.20 × 25.40 mm), and Netlist_PCB1_2026-06-28.tel.

✔ Netlist verified against the as-built board (2026-07-03). The June 28 netlist was cross-checked net-by-net against the flying-probe export: all 57 shared nets match member-for-member. The four discrepancies previously found in the obsolete June 24 netlist are confirmed resolved in the current one — discard the June 24 file. The corrections now verified in both netlist and fab data:

| Item | Obsolete netlist 2026-06-24 | Netlist + fab data 2026-06-28 (verified) |
| --- | --- | --- |
| J5 pin 5 | IO17 | IO39 (IO17 is no-connect) |
| J4 pins 1/2, J5 pins 1/2 | 1 = 3V3, 2 = GND | 1 = GND, 2 = 3V3 |
| J4 pins 7/8 | 7 = GND, 8 = IO16 | 7 = IO16, 8 = GND |
| FPC1 pin 12 (BS2) | 3V3 | GND (correct for 4-wire SPI) |

Also confirmed: all no-connect pins agree (IO3/17/38/40/41/42/45/46/47, FPC1.7, FPC1.20, USB-C SBU, U1.4, U3.2 DOUT, U6.4), and net RGB_LED_DIN (U2.25 → U3.4) is simply auto-renamed NET_10 in the fab export — same connectivity. One schematic housekeeping item: LED3 (WS2812B-V6) is an orphan symbol — present in the schematic with zero connections, absent from BOM, pick-and-place, and the board. Harmless to this build; delete it from the schematic.

# 2.  Required Equipment

| # | Item | Used for |
| --- | --- | --- |
| 1 | DMM with diode mode | Everything |
| 2 | Bench PSU, 0–5 V, adjustable current limit (CC mode) | First power, battery emulation, brownout sweep |
| 3 | USB-C cables: one A-to-C, one C-to-C | USB + CC resistor validation |
| 4 | USB power meter (inline V/A) | Input current, charge current |
| 5 | 1S Li-Po with JST-XH plug (~500 mAh) — verify pigtail polarity first | Battery tests |
| 6 | µA-capable current meter (PPK2 / µCurrent / DMM µA range) | Sleep current |
| 7 | Oscilloscope (optional but recommended) | 3V3 droop during WiFi TX, switchover transients |
| 8 | Thermal camera or IR thermometer (optional) | LDO / charger thermal |
| 9 | Qwiic SHT4x breakout + cable | I2C port test |
| 10 | VG-2864KSWEG05 OLED panel | Display tests |
| 11 | USB-UART adapter (3.3 V) | UART0 header test |
| 12 | Known-good reference board (ESP32-C6-DevKitC-1 on hand; an S3 devkit is a fairer RF reference if available) | Comparative RF test |
| 13 | Phone with nRF Connect (Nordic); a 2.4 GHz WiFi AP; PC with Arduino IDE (ESP32S3 Dev Module, 8 MB flash, USB CDC On Boot: Enabled) | RF + system tests |

# 3.  Safety Notes

- Li-Po: JST-XH battery pigtail polarity is not standardized between vendors. Verify with the DMM that battery + lands on J2 pin 1 (board position x=68.7, y=22.4 mm, net through Q3 to BATT+) before ever plugging in. Q3 provides reverse protection, but do not rely on it.

- Use the bench PSU in current-limit mode for every “first” — first power, first battery connect, first OLED attach.

- Never hot-plug the OLED FPC. Power off, seat the flex fully, close the latch, then power.

- The board has unprotected 0.5 mm-pitch FPC and USB-C pins — handle with ESD precautions.

# 4.  Board Reference Data

## 4.1  Power Tree (as built)

USB-C J1 VBUS ─(no series fuse)─► V_USB ─D3 (SS34)─► V_SYS ─U6 AP2112K-3.3─► 3V3   (EN tied to VIN)

├─ U5 MCP73831 ─► BATT+ ─ Q3 (AO3415, rev-pol) ─ J2.1 (battery +)

│     (PROG R5 = 5.1k ≈ 196 mA)   └─ R10/R11 (100k/100k) ─► VBAT_SENSE → GPIO4

├─ R15/R16 (100k/100k) ─► VUSB_SENSE → GPIO2

└─ Q1 gate (AO3407): USB present → Q1 OFF (battery isolated). USB absent (R7 10k) → Q1 ON, BATT+ → V_SYS

+5V header J5.12 ─F1 (0.5A polyfuse)─ D2 (SS34) ─► V_SYS  (input only; D2 blocks output)

Battery low side: J2.2 (BATT−) ─ Q2 FS8205A dual FET ─ GND, controlled by U1 DW01A

LED2 (green) = 3V3 power indicator via R34 2.2k. LED1 (yellow) = charge status from U5 STAT via R6 470Ω. D1 USBLC6-2SC6 = ESD clamp on USB D+/D− (rail pin tied to 3V3).

## 4.2  Expected Voltages Summary

| Net | Condition | Expected | Where to probe (side) |
| --- | --- | --- | --- |
| V_USB | USB-C 5 V source | 4.75–5.25 V | D3 anode pad, D3.2 (B) |
| V_SYS | USB powered | V_USB − 0.2…0.45 V (≈ 4.55–4.8 V) | D2/D3 cathode pad, D2.1 (B) |
| V_SYS | Battery only | V_BAT − <0.05 V (Q1 on) | same |
| 3V3 | any | 3.25–3.35 V | J4.2 / J4.12 / J5.2 (headers) |
| BATT+ | battery connected | 3.0–4.23 V | C3.2 pad (B) |
| BATT+ | charging, near full | 4.168–4.232 V (MCP73831 VREG ±0.75 %) | C3.2 |
| +5V (J5.12) | USB powered, nothing on pin | ≈ 0 V / floating (input-only pin) | J5.12 |
| VBAT_SENSE | battery connected | BATT+ ÷ 2 | U2 pin 4 (T) |
| VUSB_SENSE | USB powered | V_USB ÷ 2 (≈ 2.5 V) | U2 pin 38 (T) |
| EN | idle | ≈ 3.3 V (R12 10k pull-up) | U2 pin 3 (T) |
| IO0 | idle | ≈ 3.3 V (internal pull-up only) | U2 pin 27 (T) |
| I2C SDA/SCL | idle | ≈ 3.3 V (R9/R13 4.7k pull-ups) | J3.3 / J3.4 |
| OLED VCC | display on | 6.0–7.7 V (internal charge pump) — record | C21.2 pad (B) |
| OLED VCOMH | display on | 0.6–0.9 × VCC | C20.1 pad (B) |

## 4.3  Module Pin Map (ESP32-S3-WROOM-1, as built)

| Mod pin | GPIO | Net | Destination |
| --- | --- | --- | --- |
| 3 | EN | EN | SW1 reset btn, R12 10k→3V3, C11 1µ + C14 100n |
| 4 | IO4 | VBAT_SENSE | R10/R11 divider (ADC1_CH3) |
| 5/6/7 | IO5/6/7 | IO5/6/7 | J4.3 / J4.4 / J4.5 |
| 8/9 | IO15/16 | IO15/16 | J4.6 / J4.7 |
| 10 | IO17 | — | no connect |
| 11 | IO18 | I2C_SCL | J3.4 (Qwiic SCL), R13 4.7k PU |
| 12 | IO8 | I2C_SDA | J3.3 (Qwiic SDA), R9 4.7k PU |
| 13/14 | IO19/IO20 | D− / D+ | USB via D1 ESD |
| 15/16 | IO3/IO46 | — | no connect (strapping, internal defaults OK) |
| 17 | IO9 | OLED_DC | FPC1.15 |
| 18 | IO10 | OLED_CS | FPC1.13 |
| 19 | IO11 | OLED_MOSI | FPC1.19 (D1) |
| 20 | IO12 | OLED_SCLK | FPC1.18 (D0) |
| 21 | IO13 | IO13 | J5.4 |
| 22 | IO14 | OLED_RST | FPC1.14 |
| 23 | IO21 | IO21 | J5.10 |
| 24/26 | IO47/IO45 | — | no connect |
| 25 | IO48 | (RGB data) | U3 SK6812 DIN |
| 27 | IO0 | IO0 | SW4 boot btn (internal PU only) |
| 28/29/30 | IO35/36/37 | IO35/36/37 | J5.9 / J5.8 / J5.7 (free on N8 — no PSRAM) |
| 31/33/34/35 | IO38/40/41/42 | — | no connect |
| 32 | IO39 | IO39 | J5.5 (JTAG MTCK default — fine as GPIO with USB-JTAG) |
| 36 | IO43 (U0TXD) | “RX” | J4.10 |
| 37 | IO44 (U0RXD) | “TX” | J4.9 |
| 38 | IO2 | VUSB_SENSE | R15/R16 divider (ADC1_CH1) |
| 39 | IO1 | IO1 | J5.3 (ADC1_CH0) |

⚠ UART nets are host-perspective: the net named “TX” (J4.9) lands on the ESP32’s receive pin (U0RXD/GPIO44); “RX” (J4.10) is the ESP32’s transmit (U0TXD/GPIO43). Verify the silkscreen (MCU-06) and document unambiguously in the product manual.

## 4.4  Connector Pinouts (as built)

J4 (bottom edge, y = 1.3 mm, left→right): 1 GND · 2 3V3 · 3 IO5 · 4 IO6 · 5 IO7 · 6 IO15 · 7 IO16 · 8 GND · 9 “TX”(→U0RXD/IO44) · 10 “RX”(→U0TXD/IO43) · 11 GND · 12 3V3

J5 (top edge, y = 24.2 mm, left→right): 1 GND · 2 3V3 · 3 IO1 · 4 IO13 · 5 IO39 · 6 GND · 7 IO37 · 8 IO36 · 9 IO35 · 10 IO21 · 11 GND · 12 +5V (input only)

J3 Qwiic (BM04B-SRSS): 1 GND · 2 3V3 · 3 SDA(IO8) · 4 SCL(IO18) — standard Qwiic order (cable: black, red, blue, yellow).

J2 battery (JST-XH 2P): 1 = BAT+ (via Q3) · 2 = BAT− (via Q2/DW01A to GND). Pin 1 at x=68.7, y=22.4 mm.

J1 USB-C (TYPE-C-31-M-12, USB 2.0): VBUS→V_USB, CC1/CC2 each 5.1 k to GND (R4/R3), D±→D1→IO19/IO20, SBU floating, shell GND.

## 4.5  OLED FPC (FPC1, 30-pin 0.5 mm, VG-2864KSWEG05, 4-wire SPI)

| FPC pin | Signal | Connection |
| --- | --- | --- |
| 1, 30 | GND guard | GND |
| 2–3 / 4–5 | C2P/C2N, C1P/C1N | C16 1µF, C17 1µF (charge pump) |
| 6 | VDDB (DC/DC in) | 3V3 |
| 7 | NC | — |
| 8 | VSS | GND |
| 9 | VDD (logic) | 3V3 |
| 10/11/12 | BS0/BS1/BS2 | GND / GND / GND (4-wire SPI) |
| 13 | CS# | IO10 |
| 14 | RES# | IO14 |
| 15 | D/C# | IO9 |
| 16/17 | R/W#, E/RD# | GND |
| 18 | D0 (SCLK) | IO12 |
| 19 | D1 (MOSI) | IO11 |
| 20 | D2 | floating (correct for serial) |
| 21–25 | D3–D7 | GND |
| 26 | IREF | R14 390 k → GND |
| 27 | VCOMH | C20 4.7 µF → GND |
| 28 | VCC (panel HV) | C21 2.2 µF → GND |
| 29 | VLSS | GND |

## 4.6  Probe Point Quick Reference

Best physical probe pad per key net (from the flying-probe export; origin = board lower-left, top view; B = bottom side).

| Net | Probe at | Side | X, Y (mm) | Notes |
| --- | --- | --- | --- | --- |
| GND | J1 shell / any header GND pin | T | — | 176 pads on net |
| V_USB | D3 anode (D3.2) | B | 63.0, 10.5 | 2.0 mm pad |
| V_SYS | D2 cathode (D2.1) | B | 48.3, 23.8 | 2.0 mm pad |
| 3V3 | J4.2 header pin | T | 9.9, 1.3 | also J4.12, J5.2 |
| BATT+ | C3 + terminal (C3.2) | B | 69.5, 14.1 | 47 µF bulk cap |
| BATT− | J2.2 | T | 68.7, 19.8 | cell negative (≠ GND) |
| +5V in | J5.12 | T | 35.3, 24.2 | before F1 |
| EN | U2 pin 3 | T | 10.1, 3.7 |  |
| IO0 | U2 pin 27 | T | 24.1, 21.5 |  |
| VBAT_SENSE | U2 pin 4 | T | 11.4, 3.7 |  |
| VUSB_SENSE | U2 pin 38 | T | 10.1, 21.5 |  |
| SDA / SCL | J3.3 / J3.4 | T | 71.3, 20.5 / 19.5 |  |
| USB D+ / D− | D1.1 / D1.3 | B | 64.5, 3.4 / 1.5 | connector side |
| Charger STAT | U5 pin 1 | B | 55.6, 21.7 |  |
| Charger PROG | U5 pin 5 | B | 55.6, 24.0 | 1.0 V during charge |
| OLED VCC | C21 (pin 28 side) | B | 57.7, 16.6 | ~7 V, display on |
| OLED VCOMH | C20 (pin 27 side) | B | 56.3, 16.6 |  |
| OLED IREF | FPC1.26 | B | 56.2, 14.7 | 390 k to GND |
| RGB data | U3.4 | T | 32.8, 14.4 | IO48 |

# 5.  VIS — Incoming Visual & Mechanical Inspection

Per board, under magnification. JLCPCB AOI catches most of this; you are double-checking the failure-prone spots.

| ID | Check | Expected | P/F |
| --- | --- | --- | --- |
| VIS-01 | Overall solder quality: tombstones, missing parts, obvious bridges | None |  |
| VIS-02 | D2, D3 polarity: cathode band toward V_SYS side (D2 cathode pad at x=48.3; D3 cathode toward board interior) | Bands correct on both |  |
| VIS-03 | Pin-1 orientation vs silk: U1, U5, U6, D1, Q1, Q2, Q3 | All correct |  |
| VIS-04 | ESP32 module: aligned, castellations wetted, antenna overhangs the edge cleanly — no solder, labels, or debris under/over the antenna | Clean |  |
| VIS-05 | USB-C J1: all 16 pins wetted, no bridges, shell anchors soldered | Clean |  |
| VIS-06 | FPC1: 0.5 mm pins no bridges, actuator latch intact and opens/closes | Clean |  |
| VIS-07 | J2 battery conn: seated flat, through-hole joints filled; J4/J5 headers straight | Clean |  |
| VIS-08 | SW1/SW4 buttons click mechanically | Both click |  |
| VIS-09 | Board: no deep scratches over traces, mousebite/edge quality acceptable for product | OK |  |
| VIS-10 | Connect the OLED screen + frame; check fit and seating | Seats cleanly, frame flush |  |
| VIS-11 | Plug the board into a standard breadboard; check fit | Seats across the center channel on 2.54 mm centers |  |

# 6.  PRE — Unpowered Electrical Checks (DMM)

No power, no battery, no OLED attached. In resistance mode, capacitor charging makes readings climb — note the settled trend, not the first flash.

| ID | Measurement | Expected | Measured | P/F |
| --- | --- | --- | --- | --- |
| PRE-01 | V_USB → GND resistance (probe D3.2) | ≈ 9–10 kΩ (R7 ∥ R15+R16), not < 1 kΩ |  |  |
| PRE-02 | V_SYS → GND (probe D2.1) | High / climbing (> 50 kΩ settled) |  |  |
| PRE-03 | 3V3 → GND (probe J4.2) | > 5 kΩ and climbing, no short |  |  |
| PRE-05 | +5V pin (J5.12) → GND | > 10 kΩ (via F1+D2 into V_SYS network) |  |  |
| PRE-06 | Diode mode: red on D3.2 (V_USB), black on D2.1 (V_SYS) | 0.15–0.40 V (SS34 forward) |  |  |
| PRE-07 | Diode mode reversed: red on V_SYS, black on V_USB | OL |  |  |
| PRE-08 | Diode mode: red on J5.12, black on V_SYS | 0.15–0.45 V (F1 + D2 forward) |  |  |
| PRE-09 | USB D+ ↔ D− (D1.1 ↔ D1.3) | OL / > 1 MΩ, no short |  |  |
| PRE-10 | USB VBUS ↔ D+ and VBUS ↔ D− | No short |  |  |
| PRE-11 | J2.1 ↔ J2.2 (battery pins) | No short (> 100 kΩ) |  |  |
| PRE-12 | Diode mode: red on C3.2 (BATT+), black on J2.1 | ≈ 0.4–0.7 V (Q3 body diode) |  |  |
| PRE-13 | EN → GND, IO0 → GND | No short |  |  |

# 7.  PWR — First Power-Up and Power Rails

PWR-01 · Smoke test (bench supply, current-limited). No battery, no OLED. Bench PSU set to 5.00 V, current limit 100 mA, + to J5.12, − to J5.11 (protected by F1 and D2 — the safest entry point).

| Check | Expected | Measured | P/F |
| --- | --- | --- | --- |
| Supply current after 5 s | 15–90 mA, stable (≈ 20 mA typical on this build; may idle in download mode on blank flash) |  |  |
| Current limit NOT hit | No CC indication |  |  |
| Nothing warm to touch after 60 s (U6, U5, module) | < warm |  |  |
| Green LED2 | ON |  |  |

PWR-02 · Rail voltages (still on bench 5 V input).

| Rail | Expected | Measured | P/F |
| --- | --- | --- | --- |
| V_SYS (D2.1) | 5.0 − (F1+D2 drop) ≈ 4.5–4.8 V |  |  |
| 3V3 (J4.2) | 3.25–3.35 V |  |  |
| EN (U2.3) | ≈ 3V3 |  |  |
| BATT+ (C3.2) | ~0 V or drifting (no battery — charger unpowered on this path, OK) |  |  |

PWR-03 · USB power path. Disconnect bench supply. Plug USB-C from a 5 V / ≥1 A source through the USB power meter.

| Check | Expected | Measured | P/F |
| --- | --- | --- | --- |
| V_USB (D3.2) | 4.75–5.25 V |  |  |
| V_SYS | V_USB − 0.2…0.45 V |  |  |
| 3V3 | 3.25–3.35 V |  |  |
| Input current, idle (blank flash) | 15–90 mA (≈ 20 mA typical) |  |  |
| VUSB_SENSE (U2.38) | V_USB ÷ 2 ± 3 % |  |  |

PWR-04 · USB-C orientation / cable matrix. Validates CC pull-downs R3/R4 (5.1 k). Board must power in all four cases; C-to-C failing in both orientations while A-to-C works would indicate a CC-resistor fault.

| Case | Powers? |
| --- | --- |
| A-to-C cable, plug orientation 1 |  |
| A-to-C cable, plug flipped |  |
| C-to-C cable, orientation 1 |  |
| C-to-C cable, flipped |  |

PWR-05 · 3V3 static load regulation. USB powered. Apply load across 3V3 (J4.2) and GND (J4.1): 22 Ω/2 W ≈ 150 mA, then 11 Ω/2 W ≈ 300 mA.

| Load | 3V3 expected | U6 temp after 2 min | P/F |
| --- | --- | --- | --- |
| 150 mA | ≥ 3.25 V |  |  |
| 300 mA | ≥ 3.20 V |  |  |

Note: from USB, U6 drops ≈1.4 V. At 300 mA that is ~0.42 W in a SOT-25 — expect it hot (can exceed 60–80 °C). Sustained >300 mA average from USB is a thermal watch item; from battery (drop ≈0.4 V) it is a non-issue. Record temps.

PWR-06 · 3V3 dynamic droop during WiFi TX (scope). Flash a continuous-TX WiFi sketch (ping/UDP flood). Scope on 3V3 at C12, AC-coupled.

| Check | Expected | Measured | P/F |
| --- | --- | --- | --- |
| Droop during TX bursts | < 150 mVpp; absolute min > 3.0 V |  |  |
| No brownout resets in serial log over 5 min | None |  |  |

PWR-07 · Minimum battery voltage / brownout sweep. Bench PSU as battery on J2 (correct polarity!), current limit 800 mA, USB disconnected. Run the WiFi TX sketch. Step 4.2 → 3.0 V in 0.1 V steps, 30 s each.

| Check | Expected | Measured | P/F |
| --- | --- | --- | --- |
| Lowest supply voltage with no reset/brownout | ≤ 3.3 V (target: WiFi stable at 3.3 V) |  |  |
| Voltage where board browns out | record |  |  |

Firmware low-battery cutoff should sit ~0.1–0.2 V above the measured brownout point.

PWR-08 · Quiescent / deep-sleep current (battery). Flash a deep-sleep sketch (sleep 60 s). Battery or bench-as-battery on J2, µA meter in series in the + lead. Wait for sleep entry.

| Check | Expected | Measured | P/F |
| --- | --- | --- | --- |
| Sleep current @ 3.8 V | ≈ 0.6–0.9 mA — dominated by green LED2 (~0.6 mA); remainder AP2112 Iq ~55 µA + R10/R11 divider ~19 µA + DW01A ~3 µA + ESP ~10 µA + SS34 reverse leakage |  |  |
| (optional) Same with LED2 lifted | < 120 µA |  |  |

If sub-100 µA standby ever becomes a product goal, LED2 and the always-connected sense divider are the items to revisit — record the baseline now.

PWR-09 · +5V pin is input-only. USB powered, nothing on J5.12.

| Check | Expected | Measured | P/F |
| --- | --- | --- | --- |
| Voltage at J5.12 | ≈ 0 V / floating (D2 blocks reverse; small leakage OK) |  |  |

Document in the product manual: pin 12 is a 5 V input (fused 0.5 A), not a 5 V output.

# 8.  CHG — Battery Charger (MCP73831-2AC, PROG = 5.1 k)

Use a partially discharged Li-Po (3.6–3.9 V). USB powered. Measure battery current with the meter in series at J2 + lead, or as a USB-input-current delta.

| ID | Check | Expected | Measured | P/F |
| --- | --- | --- | --- | --- |
| CHG-01 | Fast-charge current into battery | 1000 V ÷ 5.1 kΩ ≈ 196 mA; accept 170–240 mA |  |  |
| CHG-02 | PROG pin (U5.5) voltage during fast charge | ≈ 1.0 V |  |  |
| CHG-03 | Yellow LED1 during charge | ON |  |  |
| CHG-04 | BATT+ plateau near end of charge | 4.168–4.232 V |  |  |
| CHG-05 | Termination: charge current at end | < ~20 mA, then LED1 OFF |  |  |
| CHG-06 | No battery connected, USB in | LED1 off or faint blink (charger cycling); BATT+ shows ~4.2 V pulses |  |  |
| CHG-07 | Charge + system running (WiFi demo): total USB input current | Record; can exceed 500 mA → product docs should specify a ≥1 A USB source |  |  |
| CHG-08 | U5 temperature during fast charge at VBAT ≈ 3.6 V | Warm is normal (~0.3 W); internal thermal regulation may taper — record temp |  |  |

# 9.  BAT — Battery Protection and Power Path

Bench PSU emulates the battery for threshold tests (current limit 500 mA unless noted).

| ID | Procedure | Expected | Measured | P/F |
| --- | --- | --- | --- | --- |
| BAT-01 | Reverse polarity: bench supply on J2 REVERSED, 4 V, limit 100 mA | Current ≈ 0, board dead, nothing heats; normal function after reconnecting correctly (Q3 blocks) |  |  |
| BAT-02 | Under-voltage cutoff & recovery: supply as battery, board running (no USB). Lower slowly to 2.3 V, then raise back to ≈ 3.0 V | DW01A cuts discharge at 2.4 ± 0.1 V → board powers off; recovers when raised to ≥ 3.0 V (may need a USB blip to re-enable) |  |  |
| BAT-05 | Switchover, battery → USB: run demo on battery, plug USB. Watch serial log / uptime | No reset. Q1 turns off, D3 takes over; V_SYS steps up to ~4.6 V |  |  |
| BAT-06 | Switchover, USB → battery: unplug USB while running | No reset; V_SYS drops to VBAT. Scope V_SYS (optional): dip stays > 3.0 V |  |  |
| BAT-07 | Backfeed check: USB in, no battery — voltage at J2.1 | Charger voltage (~4.2 V region, ≤ 4.4 V, may cycle) — normal; confirm NOT 5 V |  |  |

# 10.  MCU — Processor Bring-Up

| ID | Procedure | Expected | Measured | P/F |
| --- | --- | --- | --- | --- |
| MCU-01 | Plug USB to PC, blank flash | Enumerates as Espressif USB JTAG/serial debug unit (VID 303A, PID 1001) — blank flash drops the ROM into download mode automatically |  |  |
| MCU-02 | Manual download mode: hold BOOT (SW4), tap RESET (SW1), release BOOT | Re-enumerates in download mode |  |  |
| MCU-03 | esptool flash_id / chip info; read MAC | ESP32-S3, flash size 8 MB (N8); record MAC → write on board label as S/N |  |  |
| MCU-04 | Flash DVT helper firmware @ 921600 over native USB | Flash + verify OK |  |  |
| MCU-05 | Serial monitor over native USB CDC (USB CDC On Boot: Enabled) | Boot log + helper menu visible |  |  |
| MCU-06 | UART0 via header: 3.3 V USB-UART adapter, GND→J4.1; boot log expected on J4.10 (“RX” net = ESP U0TXD) at 115200; adapter TX → J4.9 | Boot log readable; note which silk label sits on which pin, record for the manual |  |  |
| MCU-07 | RESET (SW1) while running | Clean reset, boot log; no hang (EN RC ≈ 11 ms from R12·C11) |  |  |
| MCU-08 | Timekeeping sanity: log millis() for 10 min vs wall clock | Drift < 0.5 % (crystal gross check; NTP covers accuracy in product) |  |  |

# 11.  GPIO — I/O Verification

Header GPIOs on the fabbed board: J4: IO5, IO6, IO7, IO15, IO16 · J5: IO1, IO13, IO39, IO37, IO36, IO35, IO21.

| ID | Procedure | Expected | Measured | P/F |
| --- | --- | --- | --- | --- |
| GPIO-01 | Output walking test (helper 1): each header pin driven HIGH 1 s / LOW 1 s in sequence; measure at the header pin (proves pad→trace→header path). Record worst V_high and V_low across all 12 pins | Every pin ≥ 3.1 V driven HIGH and ≤ 0.1 V driven LOW |  |  |
| GPIO-02 | Input test (helper 2): internal pull-ups enabled; jumper each header GPIO to GND through 1 k | All 12 read 1 (pull-up), 0 when grounded |  |  |
| GPIO-03 | Adjacent-bridge test (helper 3): alternating pattern driven and read back | Read-back matches driven pattern (catches neighbor bridges) |  |  |
| GPIO-04 | Strapping sanity: IO0 idle (internal pull-up only); cold-boot to app 10× with no buttons | IO0 ≥ 3.0 V; 10/10 clean cold boots |  |  |

# 12.  I2C — Qwiic Port (J3)

| ID | Check | Expected | Measured | P/F |
| --- | --- | --- | --- | --- |
| I2C-01 | Idle levels SDA (J3.3), SCL (J3.4) | ≈ 3.3 V (R9/R13 4.7 k pull-ups) |  |  |
| I2C-02 | 3V3 at J3.2 | 3.25–3.35 V |  |  |
| I2C-03 | Bus scan with SHT4x on a Qwiic cable (helper 4) | Device found at 0x44 (SDA=IO8, SCL=IO18) |  |  |
| I2C-04 | Sensor read (temp / RH) | Plausible indoors: temp 10–40 °C, RH 10–90 % |  |  |
| I2C-05 | Bus at 400 kHz (helper 4 second pass) | Scan + read still OK |  |  |
| I2C-06 | Hot-plug sensor while running | Recovers on next poll, no crash |  |  |

# 13.  OLED — Display Subsystem (VG-2864KSWEG05, 4-wire SPI)

Power off before seating the flex. Insert fully, square, close the latch. First power-up with OLED: a source that lets you observe current.

| ID | Check | Expected | Measured | P/F |
| --- | --- | --- | --- | --- |
| OLED-01 | Current delta at power-on with panel attached, before init | +0–3 mA (panel idle, pump off) — a jump of tens of mA before init suggests a mis-seated/reversed flex → power off |  |  |
| OLED-02 | U8g2 init (SSD1306 128×64, 4W HW SPI: CS=10, DC=9, RST=14, SCLK=12, MOSI=11) + text | Text renders, correct orientation |  |  |
| OLED-03 | All-pixels-ON pattern | Uniform, no missing rows/columns (proves all COM/SEG through FPC) |  |  |
| OLED-04 | Border + crosshair 1-px pattern | Crisp single-pixel lines, all edges present |  |  |
| OLED-05 | Contrast sweep 0→255 | Smooth brightness ramp, no flicker |  |  |
| OLED-09 | SPI clock at target speed (U8g2 default; SSD1306 max 10 MHz) | Stable rendering 10 min, no artifacts |  |  |
| OLED-10 | Cold boot straight into demo screen ×5 | Renders every time (init reliability) |  |  |

# 14.  RGB — SK6812 (GPIO48, powered from 3V3)

Use core 3.x built-in rgbLedWrite() / neopixelWrite().

| ID | Check | Expected | Measured | P/F |
| --- | --- | --- | --- | --- |
| RGB-01 | Red, Green, Blue at brightness 64, 2 s each | Correct pure colors, correct order |  |  |
| RGB-02 | White full (255,255,255) briefly | White, no color tint/flicker (3.3 V data + 3.3 V VDD margins OK) |  |  |
| RGB-04 | Temperature-map colors from demo (cold→blue … hot→red) | Match spec |  |  |

# 15.  ADC — Sense Channels

| ID | Check | Expected | Measured | P/F |
| --- | --- | --- | --- | --- |
| ADC-01 | Bench “battery” at 3.60 V → firmware VBAT reading (GPIO4 × 2) | 3.60 V ± 5 %; record offset |  |  |
| ADC-02 | Repeat at 4.20 V and 3.20 V | ± 5 %; linearity note for one-point cal in production firmware |  |  |
| ADC-04 | VUSB reading (GPIO2 × 2): USB in | ≈ 4.75–5.25 V |  |  |
| ADC-05 | VUSB reading: USB out, on battery | < 0.5 V → firmware “USB present” flag works both ways |  |  |

# 16.  BTN / LED — Human Interface

| ID | Check | Expected | P/F |
| --- | --- | --- | --- |
| BTN-01 | SW4 (BOOT/IO0): firmware prints on press/release with 20 ms debounce | 1 idle, 0 pressed, no bounce |  |
| BTN-02 | SW1 (RESET): press while running | Immediate clean reset |  |
| LED-01 | Green LED2 | On whenever 3V3 is up (any power source) |  |
| LED-02 | Yellow LED1 matrix: charging / charge-done / no-battery / no-USB | ON / OFF / off-or-blink (record) / OFF |  |

# 17.  RF-W — WiFi

Run RF tests with the board on a non-metallic stand, away from your body, with the USB cable routed away from the antenna.

| ID | Procedure | Expected / Target | Measured | P/F |
| --- | --- | --- | --- | --- |
| RF-W-01 | WiFi scan (helper 8) | Sees all APs the reference board sees |  |  |
| RF-W-02 | Comparative RSSI: DUT vs reference board, same location, 3 m from AP, average ≥ 10 scans, repeat in 4 orientations (0/90/180/270°) | Δ RSSI ≤ 6 dB vs reference in the worst orientation; record all 4 values |  |  |
| RF-W-03 | Connect to WPA2 AP, DHCP (helper c) | Associated + IP < 10 s |  |  |
| RF-W-04 | ping ×100 from PC to board at 3 m (helper c holds the link) | Loss < 2 %; average latency 20–200 ms (~130 ms typical over WiFi) |  |  |
| RF-W-07 | Full demo (NTP + Open-Meteo fetch, plain HTTP per firmware spec) on the S3 | Weather + time render; note whether HTTPS also works on S3 (the C6 TLS hang was C6-specific — verify) |  |  |
| RF-W-08 | On battery at 3.5 V: 5 min continuous WiFi traffic (helper 9) | No brownout/reset |  |  |

# 18.  RF-B — Bluetooth LE

RainMaker BLE provisioning (former RF-B-02/03/04) was dropped from the product — the shipping demo provisions over a WiFi captive portal. Only the BLE radio check is retained for board QA.

| ID | Procedure | Expected | Measured | P/F |
| --- | --- | --- | --- | --- |
| RF-B-01 | BLE advertise (helper b); observe in nRF Connect at 1 m | Visible; RSSI recorded (compare vs reference board, same as RF-W-02) |  |  |

# 19.  SYS — System / Application Level

| ID | Procedure | Expected | Measured | P/F |
| --- | --- | --- | --- | --- |
| SYS-01 | Factory-fresh flow: erase flash → flash shipping demo → cold boot | Captive portal appears → configure WiFi → NTP sync → weather renders → RGB temp color → battery % shown |  |  |
| SYS-02 | 24 h soak on USB power, demo running, log uptime + free heap hourly | No reboot; heap stable (no leak trend) |  |  |
| SYS-04 | Power cycle ×20 (USB unplug/replug, 5 s off) | Boots to demo 20/20 |  |  |
| SYS-05 | USB data hot-plug ×10 while running on battery | No crash; CDC re-enumerates each time |  |  |
| SYS-06 | Overnight: battery connected, USB in, demo running | Morning: battery full, LED1 off, board < 45 °C, still running |  |  |
| SYS-07 | Thermal survey at steady state (WiFi demo, charging): spot-check U2 module, U5, U6 | Record temps; nothing > 85 °C surface |  |  |
| SYS-08 | Firmware pin-map migration check: sketch’s S3 pin block matches Appendix B (SDA 8, SCL 18, DC 9, CS 10, MOSI 11, SCLK 12, RST 14, RGB 48, VBAT 4, VUSB 2) | Match |  |  |

# 20.  Results Summary & Sign-Off

Executed rollup for the first-article boards (individual measurements, as logged in the companion tracker). Overall: 122 measurements, 120 pass, 2 fail (98.4 %).

| Subsystem | Measurements | Pass | Fail | N/A |
| --- | --- | --- | --- | --- |
| VIS visual | 11 | 10 | 1 | 0 |
| PRE unpowered | 12 | 12 | 0 | 0 |
| PWR power rails | 27 | 27 | 0 | 0 |
| CHG charger | 8 | 8 | 0 | 0 |
| BAT protection/path | 6 | 6 | 0 | 0 |
| MCU bring-up | 7 | 6 | 1 | 0 |
| GPIO | 6 | 6 | 0 | 0 |
| I2C | 8 | 8 | 0 | 0 |
| OLED | 7 | 7 | 0 | 0 |
| RGB | 3 | 3 | 0 | 0 |
| ADC | 5 | 5 | 0 | 0 |
| BTN | 2 | 2 | 0 | 0 |
| LED | 2 | 2 | 0 | 0 |
| RF WiFi | 8 | 8 | 0 | 0 |
| RF BLE | 1 | 1 | 0 | 0 |
| SYS | 9 | 9 | 0 | 0 |
| Total | 122 | 120 | 2 | 0 |

Issue log

| # | Test ID | Description | Severity | Disposition |
| --- | --- | --- | --- | --- |
| 1 | MCU-001 | D1 populated with SRV05-4 instead of USBLC6-2SC6; USB data pair open, board would not enumerate | Major | Board #1 reworked (bridged D+/D−); correct part specified for next batch, JLCPCB quality claim filed, verify IC marking on first articles |
| 2 | VIS-011 | Breadboard header pitch slightly wide; hard to seat across a standard breadboard | Minor | Pull the two header rows to true 2.54 mm centers next PCB revision |
| 3 | VIS-007 | J2 battery connector footprint is not the intended part | Minor | Correct the J2 connector part/footprint in the EasyEDA library and BOM before the next build |
| 4 | VIS-003 | GPIO headers (J4/J5) have no pin labels on the top silkscreen | Minor | Add J4/J5 pin-name silkscreen next revision |

Board disposition:  ☑ PASS with deviations — release candidate (two defects: D1 substitution reworked and slated for correct part next batch; header pitch/silkscreen/battery-connector fixes rolled into the next PCB revision).

Signature: ____________________     Date: ____________

# Appendix A — Open Items & DVT Findings

Entering DVT:

- UART header labeling: nets “TX”/“RX” are host-perspective (J4.9 → ESP U0RXD, J4.10 → ESP U0TXD). Verify silk in MCU-06 and document.

- Netlist rev lag — RESOLVED (2026-07-03): Netlist_PCB1_2026-06-28.tel matches the flying-probe data on all 57 nets. Remaining cleanup: delete the unconnected orphan symbol LED3 (WS2812B-V6).

- Standby current floor ≈ 0.6 mA set by green LED2 — fine for this product, but blocks any future sub-100 µA sleep claim (with SS34 leakage and the always-on 200 k sense dividers).

- AP2112 dissipation from USB at sustained >300 mA (PWR-05 thermal record decides if action is needed).

- USB current > 500 mA possible while charging + WiFi (CHG-07) → product docs to require ≥ 1 A supply.

Found in DVT (see §20 issue log / tracker):

- D1 assembly substitution (SRV05-4 vs USBLC6-2SC6) — reworked on board #1, correct part next batch, quality claim filed.

- Breadboard header pitch slightly wide; GPIO header silkscreen labels missing; J2 battery-connector footprint wrong — all rolled into the next PCB revision.

- RainMaker provisioning and Arduino OTA descoped for Rev A (product provisions over a WiFi captive portal, no cloud dependency).

# Appendix B — Verified S3 Pin Definitions (firmware migration)

// ESP32-S3 product board — fab rev 2026-06-28 (verified against flying-probe netlist)

#define PIN_I2C_SDA    8   // Qwiic J3.3, 4.7k pull-up (R9)

#define PIN_I2C_SCL   18   // Qwiic J3.4, 4.7k pull-up (R13)

#define PIN_OLED_DC    9   // FPC1.15

#define PIN_OLED_CS   10   // FPC1.13

#define PIN_OLED_MOSI 11   // FPC1.19

#define PIN_OLED_SCLK 12   // FPC1.18

#define PIN_OLED_RST  14   // FPC1.14

#define PIN_RGB       48   // SK6812 DIN

#define PIN_BOOT_BTN   0   // SW4, internal pull-up only

#define PIN_VBAT_SENSE 4   // ADC1_CH3 = BATT+ / 2 (R10/R11 100k/100k)

#define PIN_VUSB_SENSE 2   // ADC1_CH1 = V_USB / 2 (R15/R16 100k/100k)

// Free header GPIOs — J4: 5, 6, 7, 15, 16   J5: 1, 13, 39, 37, 36, 35, 21

// UART0: GPIO43 (U0TXD) on J4.10, GPIO44 (U0RXD) on J4.9

// No-connect on this board: IO3, IO17, IO38, IO40, IO41, IO42, IO45, IO46, IO47

# Appendix C — DVT Helper Firmware (spec)

One Arduino sketch, serial-menu driven over native USB CDC. Menu keys and the DVT IDs they exercise:

- 1 GPIO walking output (GPIO-01) · 2 GPIO input/pull-up report (GPIO-02, BTN-01) · 3 adjacent-bridge test (GPIO-03)

- 4 I2C scan + SHT4x 100k/400k (I2C-01..06) · 5 OLED pattern cycle (OLED-02..05/09) · 6 RGB R/G/B/W (RGB-01/02/04)

- 7 ADC dump VBAT×2 / VUSB×2 (ADC-01/02/04/05) · 8 WiFi scan + RSSI average (RF-W-01/02) · c connect + hold / ping (RF-W-03/04)

- 9 continuous WiFi TX flood (PWR-06/09, RF-W-08) · h HTTP throughput (optional sanity, not a Rev A gate) · s deep-sleep 60 s (PWR-08)

- b BLE advertise (RF-B-01) · t timekeeping 10 min (MCU-08) · i board info / MAC · w set WiFi credentials (NVS) · m menu

Build settings: ESP32S3 Dev Module, 8 MB flash, partition “8M with spiffs (3 MB APP / 1.5 MB SPIFFS)”, USB CDC On Boot: Enabled, PSRAM Disabled. Library: U8g2; everything else ships with arduino-esp32 core 3.x.